APT SURVEY REPORT ON

SPECTRUM USAGE FOR UTILITY RADIOCOMMUNICATION SYSTEMS IN ASIA-PACIFIC COUNTRIES

Edition: September 2026

The 37th Meeting of APT Wireless Group

7 – 11 September 2026

Dhaka, Bangladesh (Hybrid)

(Source: AWG-37/OUT-14)

No. APT/AWG/REP-164

APT SURVEY REPORT ON

SPECTRUM USAGE FOR UTILITY RADIOCOMMUNICATION SYSTEMS IN

ASIA-PACIFIC COUNTRIES

## 1.	Introduction

With the increasing digitalization of utility sectors such as electricity, water, gas, and transport systems, reliable and secure wireless communication systems are essential for monitoring, control, automation, and emergency response.

APT Wireless Group (AWG) conducted a survey among APT Members to collect information on spectrum usage and related radiocommunication systems for utility communications in the Asia-Pacific region.

This report consolidates the information from APT Members and is intended to provide a regional overview of current deployments, planned developments, technologies, use cases and other relevant information.

## 2.	Objective and Scope

The objective of this report is to consolidate information provided by APT Members on:

- Current and planned spectrum usage for utility communications

- Wireless technologies and standards being deployed or planned;

- Deployment status and key use cases for utility communication systems;

- Additional relevant information, case studies and national initiatives.

The scope of this report includes utilities such as:

- Electricity (Smart Grid, AMI, SCADA) ;

- Water and gas networks;

- Public transport systems (rail, metro, bus) ;

- Smart city infrastructure;

- Emergency and critical infrastructure communications;

- Other relevant utility applications.

## 3. 	Survey Responses

Information from the following APT Members are reflected in this report: Bangladesh, China, Japan, Malaysia, Solomon Islands, Sri Lanka, Thailand, Viet Nam.

3.1 Deployment Status of Utility Communication Systems

| Country | Electricity (Smart Grid, AMI, SCADA) | Water and gas networks | Public transport systems (rail, metro, bus) | Smart city infrastructure | Emergency and critical infrastructure communications | Any other |
| --- | --- | --- | --- | --- | --- | --- |
| Bangladesh | Implemented | Implemented | Implemented Pilot/Trial phase Planned | Implemented | Implemented Planned |  |
| China | Implemented | Implemented |  | Not considered |  | Not considered |
| Japan | Implemented | Implemented | Implemented | Implemented | Implemented | Implemented |
| Malaysia | Implemented | Pilot/Trial phase | Pilot/Trial phase | Implemented | Implemented |  |
| Solomon Islands | Implemented Planned(expansion) | Implemented Planned(SCADA upgrades) | Not considered | Pilot/Trial phase Planned | Implemented Planned | Implemented Planned |
| Sri Lanka | Implemented Pilot/Trial phase Planned | Not considered | Implemented Pilot/Trial phase Planned | Implemented Pilot/Trial phase Planned | Implemented Pilot/Trial phase Planned | Not considered |
| Thailand | Implemented | Implemented | Implemented Pilot/Trial phase Planned | Implemented | Implemented Planned |  |
| Viet Nam | Implemented | Pilot/Trial phase | Implemented | Implemented | Implemented |  |

3.2 Frequency Bands Used or Planned

| Country | Spectrum band | Frequency Band(s) / Range(s) | Technologies used or planned | Comments |
| --- | --- | --- | --- | --- |
| Bangladesh | Below 1 GHz | 400 MHz – 470 MHz | Two-way Radio (Analogue/Digital Trunking) |  |
| Bangladesh | Below 1 GHz | 863 MHz – 868 MHz, 915 MHz – 925 MHz | Non-MT IoT |  |
| China | Below 1 GHz | 223-235MHz | Digital |  |
| China | Below 1 GHz | 470-510MHz | Digital |  |
| Japan | Below 1 GHz | 26.175 - 50 MHz | Digital | Rail |
| Japan | Below 1 GHz | 54 - 76 MHz | Analogue/Digital | Gas/Rail |
| Japan | Below 1 GHz | 142 – 222 MHz | Analogue/Digital | Gas/Rail |
| Japan | Below 1 GHz | 158.35 – 222 MHz | Analogue/Digital | Gas |
| Japan | Below 1 GHz | 158.35 – 222 MHz | Analogue | Rail |
| Japan | Below 1 GHz | 335.4 – 470 MHz | Analogue/Digital | Gas/Rail |
| Japan | Below 1 GHz | 466.775 – 470 MHz | Analogue/Digital | Gas/Rail |
| Japan | Below 1 GHz | 406 – 420 MHz | Analogue/Digital | Telemetry |
| Japan | Below 1 GHz | 920.5 - 928.1 MHz | AMI, Wi-SUN | Gas/Electricity |
| Malaysia | Below 1 GHz | 163.50 MHz – 164.00 MHz paired with 173.50 MHz – 174.00 MHz | VHF Telemetry |  |
| Malaysia | Below 1 GHz | 440.00 MHz – 441.00 MHz paired with 445.00 MHz – 446.00 MHz | UHF Telemetry |  |
| Malaysia | Below 1 GHz | 452.5 MHz – 457.5 MHz paired with 462.5 MHz – 467.5 MHz | IMT Systems (IMT-Advanced) |  |
| Solomon Islands | Below 1 GHz | VHF/UHF land mobile bands (exact assignments not publicly available) | Analog and digital two-way radio, telemetry, private utility communications | Used for operational communications and emergency response. |
| Sri Lanka | Below 1 GHz | 136–174 MHz | Two-Way Radio , analogue/digital private mobile radio; telemetry | Applicable and current regulatory range. TRCSLs RTTE rules clearly state 136 to 174 MHz for analogue and digital Private Mobile Radio. The NFAT also has private land-mobile allocations, in the VHF range. |
| Sri Lanka | Below 1 GHz | 148–149.9 MHz; 150.05–154 MHz; 154–156.4875 MHz | Radio telemetry; private mobile radio | The solid directly documented utility candidate is the one that TRCSL NFAT clearly lists as "Fixed Service – Radio Telemetry" and "Land Mobile – Private”, in these frequency ranges. This matches the type of communications that utility telemetry and SCADA systems use even though the NFAT does not say that a specific electricity or water company is operating on it. |
| Sri Lanka | Below 1 GHz | 162.0375–174 MHz and portions of the VHF/UHF land-mobile spectrum | Private mobile radio / two-way radio; telemetry | TRCSL NFAT identifies Land Mobile – Private, Fixed Service – Network and Fixed Service – Radio Telemetry in portions of the VHF range. |
| Sri Lanka | Below 1 GHz | 380–399.9 MHz; 410–430 MHz | TETRA, digital trunking/private mobile radio | TRCSLs RTTE rules specifically recognise TETRA handsets/equipment in the frequency bands 380–399.9 MHz and 410–430 MHz The NFAT identifies the band 410–420 MHz for fixed/mobile services and the band 420–430 MHz, for fixed/mobile plus Fixed Service – Radio Telemetry and Land Mobile – Private. |
| Sri Lanka | Below 1 GHz | 410–420 MHz | Private digital radio; telemetry/network applications | According to the National Frequency Allocation Table (NFAT) of Sri Lanka, this frequency band is allocated to the Fixed Service, Mobile Service (excluding Aeronautical Mobile Service), Fixed Service - Network, and Land Mobile - Private Service. |
| Sri Lanka | Below 1 GHz | 420–430 MHz | Radio telemetry; private mobile radio; network communications | According to the NFAT of Sri Lanka, this band allocated to the telemetry/private-mobile range. |
| Sri Lanka | Below 1 GHz | 433.05–434.79 MHz | LoRa/LoRa WAN / low-power IoT | TRCSL officially lists the frequency range 433.05 to 434.79 MHz, for Low Power Wide Area Network (LPWAN) IoT services following the EU433 channel plan. Low Power Wide Area Network (LPWAN) IoT services can support utility IoT sensor applications though they must meet the conditions. |
| Sri Lanka | Below 1 GHz | 460–470 MHz | Fixed/mobile radio; potentially private operational communications | NFAT allocates fixed/mobile services in 460–470 MHz However, the current NFAT does not identify this range specifically as utility telemetry. Therefore, it should not be reported as a confirmed utility band. |
| Sri Lanka | Below 1 GHz | 806–824 MHz | Radio Local Loop; trucking; fixed/ network communications | TRCSL NFAT identifies Radio Local Loop, Trunking etc. and Fixed Service – Network in 806–824 MHz It is therefore a potential operational/network communications band but not specifically identified by TRCSL as an electricity/water utility band. |
| Sri Lanka | Below 1 GHz | 824–890 MHz | IMT/public cellular | TRCSL NFAT identifies IMT and cellular operators in this range. Utility organizations can use cellular networks, for telemetry SCADA, AMI and backhaul where such services are commercially available. However, this should be described as IMT-not utility-exclusive spectrum. |
| Sri Lanka | Below 1 GHz | 863–868 MHz | LoRa/Lora WAN / LPWAN IoT | TRCSL clearly states that the frequency range 863–868 MHz (also known as EU863–870) is designated for LPWAN IoT services. This band could be important, for meters, sensors and other distributed utility IoT applications. |
| Sri Lanka | Below 1 GHz | 920–925 MHz | LoRa/Lora WAN / LPWAN IoT |  |
| Thailand | Below 1 GHz | 9.5-16.5 kHz | Track Circuit | Metro/rail signalling - train detection |
| Thailand | Below 1 GHz | 823.5/875 kHz | FSK Short Range Devices (balise) | Track-side beacon for train position (metro) |
| Thailand | Below 1 GHz | 9.984 MHz | Eurobalise/Transponder | Train identification at fixed points (airport APM) |
| Thailand | Below 1 GHz | 4.234/27.095 MHz | Eurobalise/Transponder | Track-side signalling beacons (metro); planned Eurobalise/telepower use |
| Thailand | Below 1 GHz | 164.275-164.450 MHz / 166.550/164.400 MHz | Two-Way Radio (analogue/digital) | Police and disaster response |
| Thailand | Below 1 GHz | 380-400 MHz / 400 MHz | TETRA / Digital Trunked Radio | Metro operation control, public safety |
| Thailand | Below 1 GHz | 430-440 MHz | NB LTE | Smart Grid |
| Thailand | Below 1 GHz | 814-819/859-864 MHz | PPDR (FDD) | Public protection and disaster relief (metro, police) |
| Thailand | Below 1 GHz | 850-1575.42 MHz | IMT, Satellite positioning (GNSS/GPS), IoT | Vehicle tracking |
| Thailand | Below 1 GHz | 800/900 MHz | GSM-R / LTE-R (planned/implementing) | Railway voice and data (planned for HSR) |
| Thailand | Below 1 GHz | 900 MHz | Two-Way Radio, NB-IoT | Emergency telephone; water network IoT |
| Thailand | Below 1 GHz | HF/VHF/UHF | Two-Way Radio (analogue/digital) | Police operations |
| Thailand | 1 GHz to 6 GHz | 1.8/2.1 GHz | NB-IoT | Water network IoT |
| Thailand | 1 GHz to 6 GHz | 1.9 GHz | 5G-FRMCS (future) | Future railway voice/data (planned) |
| Thailand | 1 GHz to 6 GHz | 2.4-2.5 GHz | CBTC (IEEE 802.11n/ac), Wi-Fi | Metro train control, onboard systems |
| Thailand | 1 GHz to 6 GHz | 5.8 GHz | CBTC, Wi-Fi, DSRC | Metro signalling; ETC Easy Pass via DSRC, |
| Thailand | 1 GHz to 6 GHz | 5.8 GHz | CBTC (IMT-Private/Captive) | CBTC for Airport Automated People Mover (APM) |
| Thailand | 1 GHz to 6 GHz | SHF C-band | Satellite communications | Police |
| Thailand | 1 GHz to 6 GHz | 1-6 GHz | GPS/DVR/body-worn camera systems | Vehicle monitoring |
| Thailand | Above 6 GHz | 24 GHz | Doppler Radar Sensor | Train detection / signalling |
| Viet Nam | Below 1 GHz | Portions of: 136-174 MHz; 406.1-450 MHz; 698-806 MHz 880-915 MHz; 935-960 MHz | Two Way Radio ( Analogue, Digital, Trunking) IMT -Public (2G/3G/4G/5G) |  |
| Bangladesh | 1 GHz to 6 GHz | 2.4 GHz / 5.8 GHz | Wi-Fi, IoT | • 2.4 GHz used for local short-range smart meter mesh networks. |
| China | 1 GHz to 6 GHz |  |  |  |
| Japan | 1 GHz to 6 GHz | 2.3 – 2.4 GHz | Digital |  |
| Malaysia | 1 GHz to 6 GHz | 1900 MHz – 1910 MHz | IMT-Railway Mobile Radio (RMR) |  |
| Malaysia | 1 GHz to 6 GHz | 2400 MHz – 2483.5 MHz (Up to 2500 MHz for UAS/Inductive) | Short-Range Device (SRD) |  |
| Solomon Islands | 1 GHz to 6 GHz | Cellular bands used by public mobile networks (3G/4G/LTE) | IMT Public Networks, SCADA telemetry, IoT applications | Water and power utilities increasingly use cellular connectivity for monitoring and control. |
| Sri Lanka | 1 GHz to 6 GHz | 1710–1785 MHz / associated IMT allocations | IMT-public 4G/5G | Public cellular spectrum can support utility communications, such as data and telemetry but it is not specifically set aside for utility use. TRCSLs RTTE rules include 4G equipment operating in the 1710–1785 MHz range. |
| Sri Lanka | 1 GHz to 6 GHz | 2300–2400 MHz | IMT / broadband; potentially private/captive applications subject to licensing | TRCSL RTTE documentation mentions equipment in the 2300–2400 MHz range. I would not call this a confirmed utility-specific band unless there is an official utility frequency assignment. |
| Sri Lanka | 1 GHz to 6 GHz | 2400–2483 MHz | Wi-Fi / IEEE 802.11; Zigbee / IEEE 802.15.4; other ISM/short-range technologies | TRCSL RTTE documentation recognises WLAN operation in 2400–2483 MHz This can support local utility networks, sensors and gateways, subject to applicable technical conditions. |
| Sri Lanka | 1 GHz to 6 GHz | 2500–2620 MHz | IMT / broadband | Recognized in TRCSL RTTE documentation for equipment. Utility use would normally be through operator networks unless a specific private assignment is made. |
| Sri Lanka | 1 GHz to 6 GHz | 3400–3600 MHz | 5G IMT; future/private-captive 5G potential | TRCSL sold the 3400 to 3500 MHz range to Dialog and the 3500 to 3600 MHz range to SLT-Mobitel for IMT-2020 (5G). TRCSL had looked at the idea of using the 3.5 GHz spectrum for private 5G networks before, but the latest decision is mainly for IMT-2020 operators. |
| Sri Lanka | 1 GHz to 6 GHz | 5.9–6.4 GHz | Microwave / point-to-point links | TRCSLs Fixed Services licensing page clearly states that microwave frequency assignment is available in the 5.9 to 6.4 GHz range. This frequency range can be important, for utility backhaul SCADA backhaul and general network connectivity needs. |
| Viet Nam | 1 GHz to 6 GHz | 1710-1785 MHz; 1805-1880 MHz; 1900-2010 MHz; 2110-2200 MHz 2400 - 2483.5 MHz | IMT -Public (2G/3G/4G/5G) |  |
| Bangladesh | Above 6 GHz |  |  |  |
| China | Above 6 GHz |  |  |  |
| Japan | Above 6 GHz | 6.57 – 6.87 GHz | Digital | Backhaul |
| Japan | Above 6 GHz | 7.425 – 7.75 GHz | Digital | Backhaul |
| Japan | Above 6 GHz | 7.75 – 7.9 GHz | Digital | Backhaul |
| Japan | Above 6 GHz | 12.2 – 12.5 GHz | Digital | Backhaul |
| Japan | Above 6 GHz | 17.82 – 17.85 GHz | Digital | Backhaul |
| Japan | Above 6 GHz | 18.57 – 18.6 GHz | Digital | Backhaul |
| Japan | Above 6 GHz | 37.5 – 37.9 GHz | Digital | Backhaul |
| Japan | Above 6 GHz | 38.5 – 38.9 GHz | Digital | Backhaul |
| Solomon Islands | Above 6 GHz | Microwave links, satellite Ku/Ka-band services | Microwave backhaul, VSAT, satellite communications | Used for connectivity to remote islands and backup communications. |
| Sri Lanka | Above 6 GHz | 7.1 GHz, 7.4 GHz, 7.9 GHz | Microwave / point-to-point links | TRCSL lists these bands under its fixed services microwave frequency assignment framework. TRCSL says that these bands are suitable for fixed wireless backhaul applications, including potential utility network backhaul. |
| Sri Lanka | Above 6 GHz | 8 GHz | Microwave / point-to-point | TRCSL lists 8 GHz microwave frequency assignments under Fixed Services. |
| Sri Lanka | Above 6 GHz | 11 GHz | Microwave / point-to-point | TRCSL lists 11 GHz under its Fixed Services microwave assignment framework. |
| Sri Lanka | Above 6 GHz | 13 GHz | Microwave / point-to-point | TRCSL lists 13 GHz under its Fixed Services microwave assignment framework. |
| Sri Lanka | Above 6 GHz | 15 GHz | Microwave / point-to-point | TRCSL lists 15 GHz under Fixed Services microwave assignment. |
| Sri Lanka | Above 6 GHz | 18 GHz | Microwave / point-to-point | TRCSL lists 18 GHz under Fixed Services microwave assignment. |
| Sri Lanka | Above 6 GHz | 23 GHz | Microwave / point-to-point | TRCSL lists 23 GHz under Fixed Services microwave assignment. |
| Sri Lanka | Above 6 GHz | 26 GHz | Microwave / high-capacity fixed links | TRCSL lists 26 GHz under Fixed Services. |
| Sri Lanka | Above 6 GHz | 27 GHz (27,200–27,400 MHz currently assigned) | 5G IMT-2020; potential high-capacity/private-industry applications in future | TRCSL assigned 27,200–27,400 MHz to Dialog for high-capacity 5G deployments in December 2025. This is not presently a utility-specific allocation. |
| Sri Lanka | Above 6 GHz | 38 GHz | Microwave / point-to-point | TRCSL lists 38 GHz under Fixed Services microwave frequency assignment. |
| Viet Nam | Above 6 GHz |  |  |  |

3.3 Key Use Cases

| Use case | Bangladesh | China | Japan | Malaysia | Solomon Islands | Sri Lanka | Thailand | Viet Nam |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Smart metering (AMI) | Supported (Massive rollout ongoing by power distributors) | Supported | Supported | Supported | Planned | Planned | Supported | Supported |
| Distribution automation | Planned | Not Supported |  | Supported | Supported | Supported | Supported | Supported |
| Wide-area monitoring (WAMS) | Planned | Not Supported |  | Supported | Planned | Not Supported | Supported | Supported |
| Outage management | Supported | Not Supported |  | Supported | Supported | Supported | Supported | Supported |
| Metering and billing | Supported (Prepaid gas and smart water meters are expanding) | Supported |  | Supported | Supported | Supported | Supported | Supported |
| Leakage detection | Planned | Not Supported |  | Supported | Supported | Not Supported | Supported | Supported |
| Pipeline monitoring | Planned | Not Supported | Supported | Supported | Supported | Not Supported | Not Supported | Supported |
| Rail communication systems | Supported | Not Supported | Supported | Supported | Not Supported | Supported | Supported | Supported |
| Metro / CBTC | Supported (Implemented in Dhaka Metro Line-6) | Not Supported | Supported | Supported | Not Supported | Not Supported | Supported | Supported |
| Fleet management | Supported | Not Supported |  | Not Supported | Planned | Supported | Supported | Supported |
| Street lighting | Supported (Piloted/Implemented in major city corporations) | Not Supported |  | Supported | Planned | Planned | Supported |  |
| Environmental monitoring | Planned | Not Supported |  | Supported | Planned | Supported | Supported | Supported |
| Waste management | Planned | Not Supported |  | Supported | Planned | Supported | Not Supported |  |
| Disaster response | Supported | Not Supported |  | Supported | Supported | Planned | Supported | Supported |
| Backup communications | Supported | Not Supported | Supported | Supported | Supported | Planned | Supported | Supported |

3.4 Additional Information, Case Studies and National Initiatives

| Country | Additional information / case studies / national initiatives |
| --- | --- |
| Bangladesh | Smart Grid Roadmap: Bangladesh power sector utilities (BPDB, DPDC, DESCO, WZPDCO, NESCO) are aggressively transitioning towards Advanced Metering Infrastructure (AMI). Millions of smart prepaid meters are currently being deployed utilizing RF Mesh (Wi-SUN compliant) and public cellular networks (4G/NB-IoT). Transport Modernization: Dhaka Mass Transit Company Limited (DMTCL) has successfully implemented modern wireless Communication-Based Train Control (CBTC) systems for Dhaka Metro Rail, operating in 2.4GHz band. |
| China | No additional information provided. |
| Japan | N/A |
| Malaysia | No additional information provided. |
| Solomon Islands | National Context The Solomon Islands is an archipelagic nation comprising nearly 1,000 islands, creating unique utility communication challenges. Remote communities, difficult terrain, limited electricity access and high infrastructure costs make wireless communications critical for utility operations and service delivery. Key National Initiatives Solomon Power SCADA Programme. Solomon Power is implementing SCADA systems to improve monitoring, control, fault management and operational efficiency of electricity networks. Solomon Water SCADA Modernisation. Solomon Water has deployed and continues to expand SCADA and telemetry systems to support water treatment, distribution monitoring, water quality management, smart water meter and operational resilience. These systems utilize remote communications and telemetry links between dispersed sites. Disaster and Emergency Communications. The National Disaster Management Office (NDMO), supported by the Emergency Telecommunications Cluster (ETC), is strengthening national radio communications and satellite communications capabilities to ensure continuity of communications during disasters. Currently, ITU is assisting Solomon Islands to develop a National Emergency Telecommunication Plan (NETP) for this commitment. Solomon Islands National Broadband Infrastructure Project (SINBIP). The Solomon Islands Government is rolling out a national communications infrastructure programme involving mobile towers, microwave links and VSAT connectivity that can support future utility communications applications in remote areas. Future Challenges Limited availability of dedicated utility spectrum. High costs associated with serving remote islands. Dependence on satellite and microwave backhaul infrastructure. Need for greater deployment of smart metering and IoT technologies. Resilience requirements due to frequent natural disasters. Need for greater integration between utility SCADA systems and national broadband infrastructure. Future Spectrum Considerations Solomon Islands may benefit from: Continued access to VHF/UHF spectrum for critical utility operations. Expansion of broadband spectrum resources supporting utility IoT. Greater use of LTE/5G private networks for critical infrastructure. Satellite-based utility monitoring and resilience applications for remote islands. Regional harmonization of utility communications spectrum and standards |
| Sri Lanka | Sri Lanka is making progress in building communications and digital infrastructure that supports utility modernization, IoT and M2M applications and resilient communications for critical services. Several national initiatives stand out as drivers of this development. Electricity – Advanced Metering Infrastructure (AMI): The Ceylon Electricity Board (CEB) is moving forward with the implementation of Advanced Metering Infrastructure (AMI). In 2025 CEB began a market-sounding exercise to set up Smart Meter Head-End Systems (HES) at Distribution Control Centers. This AMI system will allow for remote meter reading, remote control, data acquisition and real-time telemetry for electricity metering. This effort marks a step toward digitizing electricity distribution and enabling smart metering across the country. M2M / IoT connectivity: Sri Lanka’s mobile communications sector already supports Machine-to-Machine (M2M) connectivity. The Telecommunications Regulatory Commission of Sri Lanka (TRCSL) has approved M2M packages for services enabling applications such as vehicle tracking and asset monitoring. These IoT-based services provide a made commercial platform that utilities and other critical infrastructure operators can use for distributed monitoring and telemetry. National broadband connectivity – "Gamata Sannivedanaya" (Connect Sri Lanka): TRCSL is driving the "Gamata Sannivedanaya" project to expand telecommunications coverage 4G. The project will require about 573 new telecommunications tower sites to complete the planned 4G network and 103 new telecommunications tower sites already completed. This expanded connectivity will support distributed services and monitoring applications across the country. Emergency and disaster communications: In March 2026 TRCSL signed a grant agreement with the Government of Japan to establish an Amateur Radio Emergency Communication Network in Sri Lanka. The project includes HF base stations, VHF/UHF transceivers, handheld radios and necessary infrastructure. The goal is to create an independent communication network for emergency response during natural disasters when regular telecom networks may fail. Regulatory framework for utility communications: TRCSL oversees the national radio-frequency spectrum and manages licensing for radio communications. This includes telemetry, private mobile radio, IoT/LPWAN and fixed microwave services. As a result, utility and critical-infrastructure operators can deploy radio communication systems within the national spectrum framework ensuring secure and compliant operations. |
| Thailand | - |
| Viet Nam | No additional information provided. |

_______________________